# Project Handoff: GPU-Accelerated Vision-to-Action UR5e

End-to-end development plan • Hybrid ROS 2 + NVIDIA architecture

# 1. Project objective

Build a reproducible UR5e manipulation system that starts with conventional ROS 2 robotics software and progressively introduces NVIDIA acceleration only where it provides a measurable technical benefit. The final system should perceive an object, estimate its 6-DoF pose, construct/use a 3D scene, plan a collision-aware trajectory, and execute manipulation from a structured task command. The project should finish with quantitative comparisons and GPU profiling.

# 2. Final research/engineering question

How does selectively GPU-accelerated perception, 3D reconstruction, transport, inference, and motion planning affect latency, robustness, and manipulation success in a simulated UR5e system?

# 3. Architecture

Core robotics: ROS 2 + TF2 + MoveIt 2 + RViz2 + Python/C++ + OpenCV.

Simulation: NVIDIA Isaac Sim + OpenUSD + PhysX.

Perception: NVIDIA Grounding DINO → FoundationPose.

3D scene: nvBlox, introduced only for dynamic/unknown-obstacle scenarios.

Planning: MoveIt 2 with NVIDIA cuMotion as the accelerated planning backend.

GPU data path: NITROS only on high-bandwidth perception paths where it is useful.

Optimization: one meaningful CUDA component + TensorRT benchmark for one model.

Profiling: NVIDIA Nsight Systems; Nsight Compute only if the CUDA kernel warrants it.

Extensions: Omniverse Replicator and Isaac Lab after the core system is stable.

GR00T/VLA work is explicitly a stretch goal, not part of the minimum viable project.

# 4. Hardware/software strategy

Current laptop: Ubuntu 22.04.5, RTX 2050 4 GB, 16 GB RAM, ~86 GB root filesystem. Use it as the lightweight development machine. Do not force the full current Isaac ROS/Isaac Sim stack onto it. Heavy NVIDIA workloads should run on a separate supported GPU workstation/cloud instance.

Local laptop:

ROS 2 / lightweight robotics development

MoveIt 2 and RViz2

Python/C++/OpenCV

Git and documentation

small CUDA/TensorRT experiments

Remote NVIDIA machine:

Ubuntu 24.04 + supported NVIDIA GPU

Isaac Sim

Isaac ROS

FoundationPose/Grounding DINO

nvBlox/cuMotion/NITROS

TensorRT/Nsight/Replicator/Isaac Lab as needed

# 5. Phase-by-phase development plan

## Phase 0 — Repository and engineering baseline

Create repository and branch strategy.

Define package boundaries and ROS interfaces before adding perception.

Add README, architecture diagram, environment notes, issue tracker, experiment log.

Create a reproducible baseline and commit after every working milestone.

Exit criterion: clean repository with a documented UR5e development workflow.

## Phase 1 — UR5e fundamentals

Load UR5e model.

Verify joint states, TF tree, robot description and controller interfaces.

Implement forward/inverse kinematics through standard robotics tools.

Create target end-effector poses.

Plan and execute trajectories in simulation.

Visualize robot, frames and trajectories in RViz2.

Exit criterion: a commanded end-effector pose produces a valid collision-free trajectory and repeatable simulated execution.

## Phase 2 — Isaac Sim integration

Create the UR5e scene in Isaac Sim.

Add table, workspace and basic collision geometry.

Connect Isaac Sim to ROS 2.

Verify joint states, TF, command/feedback flow and clock synchronization.

Create deterministic reset/launch scripts.

Exit criterion: the same ROS 2 control workflow works against the Isaac Sim UR5e.

## Phase 3 — Motion-planning baseline and cuMotion comparison

Establish MoveIt 2 baseline planning.

Record planning latency, trajectory duration, success/failure and collision outcomes.

Integrate cuMotion.

Repeat the exact same benchmark set.

Compare results rather than assuming GPU acceleration is better.

Exit criterion: reproducible MoveIt-vs-cuMotion benchmark with plots/tables.

## Phase 4 — RGB-D perception baseline

Add simulated RGB-D camera.

Publish image/depth/camera-info topics.

Build point-cloud and depth-processing utilities with OpenCV/standard ROS tools.

Calibrate and verify camera-to-robot TF.

Start with known object identities and simple geometric pose estimates.

Exit criterion: an object pose can be transformed reliably from camera frame to robot/world frame.

## Phase 5 — FoundationPose

Introduce NVIDIA FoundationPose.

Prepare object assets/CAD/mesh inputs.

Estimate 6-DoF pose from RGB-D.

Compare FoundationPose against the simple depth/geometric baseline.

Measure position error, orientation error, latency and downstream grasp success.

Exit criterion: quantitative evidence that the NVIDIA pose estimator improves the manipulation pipeline in selected scenarios.

## Phase 6 — nvBlox and dynamic scenes

Introduce clutter and previously unknown obstacles.

Build a 3D representation with nvBlox.

Feed relevant obstacle information into the planning stack.

Test static, dynamic and partially occluded scenes.

Measure replanning latency and collision-free success.

Exit criterion: dynamic/unknown-obstacle scenarios demonstrate a measurable benefit over the static planning-scene baseline.

## Phase 7 — Grounding DINO and language-conditioned selection

Implement a small task parser for commands such as 'pick the red cube'.

Use Grounding DINO to ground the language target to an image region.

Pass the selected object to FoundationPose.

Keep language parsing modular so the perception/planning stack does not depend on one LLM.

Exit criterion: commands select the intended object across controlled multi-object scenes.

## Phase 8 — NITROS, CUDA and TensorRT optimization

Use NITROS only on high-bandwidth GPU perception paths.

Implement one custom CUDA preprocessing/geometry operation with a CPU reference implementation.

Benchmark one suitable model or inference stage with PyTorch versus TensorRT FP16.

Measure copies, latency, throughput and GPU utilization.

Exit criterion: at least one optimization produces a measured improvement or a documented negative result explaining why it was not worthwhile.

## Phase 9 — Nsight profiling and end-to-end benchmark

Profile the full perception-to-action pipeline.

Identify CPU/GPU bottlenecks, synchronization points and data-transfer overhead.

Optimize only the highest-value bottleneck.

Repeat all benchmark scenarios after optimization.

Exit criterion: before/after profiling evidence and a final performance table.

## Phase 10 — Robustness and evaluation

Create controlled scenario families: clean, cluttered, partial occlusion, multiple similar objects, dynamic obstacle and lighting/domain variation.

Run repeated trials with fixed seeds where possible.

Report detection/pose/planning/manipulation metrics separately.

Report failure modes instead of hiding them.

Exit criterion: reproducible evaluation dataset/configuration and a final results section.

## Phase 11 — Optional research extensions

Use Omniverse Replicator for randomized synthetic data/evaluation.

Use Isaac Lab for a learned manipulation policy only after the classical pipeline is stable.

Consider a GR00T/VLA experiment only as a separate stretch branch.

Exit criterion: optional extension is isolated and does not destabilize the core project.

# 6. Repository structure

ur5e-gpu-manipulation/
├── README.md
├── docs/
│   ├── architecture.md
│   ├── setup-local.md
│   ├── setup-nvidia.md
│   └── experiments.md
├── docker/
├── src/
│   ├── task_parser/
│   ├── robot_control/
│   ├── perception_interfaces/
│   ├── scene_manager/
│   ├── manipulation/
│   └── evaluation/
├── nvidia/
│   ├── grounding_dino/
│   ├── foundationpose/
│   ├── nvblox/
│   ├── cumotion/
│   ├── nitros/
│   ├── cuda/
│   └── tensorrt/
├── config/
├── launch/
├── assets/
├── usd/
├── scripts/
├── benchmarks/
│   ├── raw/
│   ├── processed/
│   └── plots/
└── results/

# 7. Metrics to collect

| Area | Metrics |
| --- | --- |
| Perception | Detection success, false selection rate, pose position error, orientation error, inference latency. |
| Planning | Planning success, planning latency, trajectory duration, path length, collision-free success. |
| System | End-to-end latency, CPU utilization, GPU utilization, GPU memory, host-device transfer/copy overhead. |
| Manipulation | Pick success, place success, total task success, recovery/replan rate. |
| Robustness | Performance versus clutter, occlusion, object similarity and dynamic obstacles. |

# 8. Required benchmark comparisons

Geometric/depth pose baseline vs FoundationPose.

MoveIt 2 baseline planner vs cuMotion.

Static planning scene vs nvBlox-based dynamic/unknown obstacle handling.

Conventional ROS 2 transport vs NITROS on a selected high-bandwidth path.

PyTorch inference vs TensorRT FP16 for one selected model/stage.

CPU preprocessing vs the custom CUDA implementation.

# 9. Definition of done

UR5e executes repeatable collision-free trajectories in Isaac Sim through ROS 2.

The system selects an object from a structured natural-language command.

FoundationPose supplies a usable 6-DoF object pose.

The planner can execute a pick/place task.

At least one dynamic/unknown-obstacle scenario is evaluated with nvBlox.

cuMotion is benchmarked against the baseline planner.

At least one CUDA/TensorRT/NITROS optimization is benchmarked.

Nsight profiling identifies at least one bottleneck.

Results are reproducible from documented configurations.

The GitHub repository contains architecture, setup, benchmark methodology, results and failure analysis.

# 10. Resume positioning

Do not present the project as 'UR5e + Isaac Sim'. The resume story should emphasize GPU-accelerated perception-to-manipulation, quantitative benchmarking, and selective optimization.

Built a language-conditioned UR5e manipulation pipeline in NVIDIA Isaac Sim with ROS 2, Grounding DINO, FoundationPose, MoveIt 2 and cuMotion.

Integrated GPU-accelerated 6-DoF perception and 3D scene reconstruction, evaluating pose accuracy, planning latency and manipulation success across controlled scenarios.

Profiled and optimized selected GPU workloads using CUDA, TensorRT/NITROS and NVIDIA Nsight Systems, reporting before/after latency and utilization.

# 11. Immediate next actions

1. Create the GitHub repository with the structure above.

2. Do not reinstall Ubuntu or add the heavy NVIDIA stack to the laptop yet.

3. Set up the local lightweight ROS 2/MoveIt development layer.

4. Obtain a supported remote/cloud NVIDIA GPU environment before installing current Isaac Sim/Isaac ROS.

5. Implement Phase 1 only: UR5e model, TF, MoveIt 2, target pose and trajectory execution.

6. Do not add perception until Phase 1 has a clean pass.

# 12. Project principle

Use NVIDIA where it creates a measurable advantage; use lightweight standard robotics software everywhere else.